Battery Fundamentals & SOC Definition

Q1: What is State of Charge (SOC), and why can't it be measured directly with a sensor?

- Answer: SOC is the ratio of remaining usable charge to nominal capacity (SOCt=SOC0-1Cnitdt). It cannot be measured directly because chemical charge storage occurs inside sealed electrodes; sensors can only observe external terminal parameters like voltage, current, and surface temperature under dynamic load distortion.

Q2: Why does terminal voltage fail as a direct indicator of SOC during active operation?

- Answer: Under load, terminal voltage (Vterm) deviates from Open Circuit Voltage (Voc) due to internal ohmic resistance drop (I⋅R0) and electrochemical charge-transfer diffusion polarization. Additionally, Li-ion batteries exhibit a flat OCV plateau in the 20%–80% SOC region, where tiny voltage changes correspond to massive charge differences.

Classical vs. Machine Learning Estimation

Q3: What are the primary failure modes of Coulomb Counting and Open Circuit Voltage (OCV) lookup?

- Answer:

  - Coulomb Counting: Open-loop integration accumulates sensor noise and DC offsets over time without error-correcting feedback. It also requires exact initial state knowledge (SOC0).

  - OCV Lookup: Requires hours of resting time for concentration gradient relaxation before accurate Voc readings can be obtained, making it unusable during active dynamic driving.

Q4: Why does Extended Kalman Filtering (EKF) drift under constant current sensor bias?

- Answer: EKF relies on Coulomb counting within its state transition model. Persistent DC sensor bias acts as an unmodeled systematic disturbance, causing the state transition step to introduce cumulative estimation drift.

Q5: Why did you select a Multi-Layer Perceptron (MLP) over Random Forest or Deep Learning models like LSTMs/CNNs?

- Answer:

  - vs. Random Forest: Random Forests produce discontinuous step outputs and require significant ROM (>120 KB for decision trees), exceeding low-cost MCU limits.

  - vs. LSTMs/CNNs: Deep recurrent/convolutional networks demand high-end accelerators, excessive RAM, and high MAC operations.

  - MLP 3-16-1: Provides smooth, continuous C∞ predictions with an ultra-low footprint (81 parameters, 348 bytes) and 6.03 μs execution latency on ESP32.

Feature Engineering & Data Preprocessing

Q6: What is the input vector to your MLP, and what is the physical rationale for including Vsmooth?

- Answer: The input vector is X=Vnoisy,Inoisy,Vsmooth.

  - V provides terminal voltage anchoring.

  - I provides flux sign and magnitude (distinguishing charge vs. discharge).

  - Vsmooth (60-second causal moving average) filters high-frequency I⋅R0 switching spikes without future data leakage, allowing the model to decouple transient ohmic drops from slow bulk SOC diffusion.

Q7: How did you handle 4-wire Kelvin probe measurements in the NASA dataset during preprocessing?

- Answer: NASA logs contain four separate sensing channels (Vbat1 through Vbat4). Taking the arithmetic mean across channels cancels out localized contact resistance variability and measurement channel noise before resampling to a uniform 5-second time grid.

Hardware Emulation & Sensor Noise

Q8: How does your digital twin model real-world hardware distortion?

- Answer: It routes clean NASA cell data through a virtual signal chain:

  - Resistor voltage divider (÷2).

  - ESP32 12-bit SAR ADC quantization (0.8 mV steps) with 5 mV RMS Gaussian noise.

  - Current sensor thermal drift simulated by a +30 mA DC offset.

  - TP4056 state machine simulating CC/CV charging transition thresholds.

Q9: Why is your ML model immune to unbounded drift from sensor DC bias?

- Answer: Unlike open-loop integrators, the MLP uses terminal voltage as a primary input feature (84.2% permutation importance). Terminal voltage is physically anchored to the thermodynamic chemical equilibrium of the cell, preventing cumulative error growth over extended operational cycles.

Embedded C Deployment & Optimization

Q10: Why did you choose tanh activation over ReLU in the hidden layer?

- Answer: ReLU uses piecewise linear planes with zero second derivatives, causing sharp slope transitions. tanh produces continuous, smooth curves that better match the chemical potential gradients of lithium intercalation and prevents dying neurons or gradient saturation issues on bounded MCU registers.

Q11: How was Python-to-C model parity verified?

- Answer: The trained model parameters were exported into zero-dependency C header files (mlp.h). Automated bitwise C unit tests (test_c_parity.c) evaluated predictions against 706 golden test vectors, confirming error matching down to 2.7×10-5%.

Q12: What is the impact of INT8 quantization on MCU performance and model accuracy?

- Answer: Quantizing FP32 floating-point weights to 8-bit signed integers reduces memory overhead and allows execution on fixed-point MCUs with only a 0.15% point accuracy penalty, confirming feasibility for low-cost 8-bit microcontrollers.

State of Health (SOH) & Aging Engine

Q13: How does your SOH engine model capacity fade vs. internal resistance growth?

- Answer:

  - Capacity Fade (SOHC): Evaluates Cactual/Cnominal using an Ah-throughput square-root power-law model (ΔC=αAhthroughput) for irreversible lithium loss.

  - Resistance Growth (SOHR): Tracks Solid Electrolyte Interphase (SEI) growth by extracting dynamic ohmic drops (ΔV/ΔI) during step current transitions, tracking internal resistance growth from 85 mΩ (fresh) toward a 200 mΩ EOL threshold.

Q14: How are SOHC and SOHR combined to project Remaining Useful Life (RUL)?

- Answer: They are combined using a weighted composite metric (SOHoverall=0.60⋅SOHC+0.40⋅SOHR) to balance energy delivery capacity and power capability. RUL projects the remaining Equivalent Full Cycles (EFC) until SOHoverall reaches the 80% End-of-Life retirement limit.

Hardware Robustness & Stress Analysis

Q15: How does varying the ADC resolution impact the model's accuracy, and why was 12-bit selected?

- Answer: The system's stress sweeps evaluated 10-bit (Arduino), 12-bit (ESP32), and 14-bit (external) ADC resolutions. The 12-bit ADC was selected because it provides an optimal cost-accuracy trade-off, achieving a 1.25% Mean Absolute Error (MAE).

Q16: What happens to the model's accuracy if cheaper resistors with higher tolerances are used in the voltage divider?

- Answer: A simulated voltage divider gain error sweep of ±2% (representing 1% versus 5% tolerance resistors) demonstrated that the model's error remains bounded within 1.6 percentage points, proving its resilience to low-cost hardware components.

Q17: How did the MLP model perform under maximum tested ADC noise and current sensor offset?

- Answer: When subjected to an ADC noise sweep up to 30 mV, the error rose gracefully from a clean 1.18% to only 1.48%. Across an extreme current sensor offset sweep from -100 mA to +100 mA, the error remained strictly bounded below 2.0%.

Web Dashboard & Telemetry

Q18: What data is transmitted in the Wi-Fi telemetry payload, and how often is it streamed?

- Answer: The virtual ESP32 streams live JSON frames via WebSockets at a frequency of 5 Hz. The payload contains the current voltage, current, estimated SOC, and charging status (e.g., {"v": 3.82, "i": -1.02, "soc": 74.2, "status": "Discharging"}).

Q19: What interactive controls does the digital twin dashboard provide for evaluating the model?

- Answer: The dashboard provides a 1x to 100x replay speed multiplier, adjustable ADC noise sliders (0-30 mV), and live current offset sliders (-100 to +100 mA). It also includes a drag-and-drop CSV parser with auto delimiter sniffing for instantaneous ground-truth error export.

Future Scope & Industry Scaling

Q20: How do you plan to handle the initialization of SOC if the initial state is completely unknown upon startup?

- Answer: The future roadmap proposes implementing a "Cold-Start Observer". This automated relaxation detector would initialize the starting SOC from static thermodynamic OCV curves before active driving commences.

Q21: How could the current machine learning approach be adapted for Lithium Iron Phosphate (LFP) batteries?

- Answer: Generalizing the model to LFP chemistries requires extending the training pipeline to handle their ultra-flat OCV plateaus. This would involve incorporating temperature sensors into the input feature vector (e.g., adding cell surface thermocouples and thermal gradients) to capture tiny environmental variations.

Q22: What is the "Industry Hybrid Pipeline" mentioned in your roadmap, and why is it beneficial?

- Answer: The hybrid architecture combines a physics-based closed-loop Kalman filter state observer with a lightweight neural network. The Kalman filter guarantees physical safety boundaries, while the neural network layer corrects residual non-linear modeling and aging errors.

Q23: How will real-time Incremental Capacity Analysis (ICA) improve the system's State of Health (SOH) tracking?

- Answer: Implementing online ICA on the ESP32 allows the system to monitor dQ/dV peak detection. This differential tracking helps accurately monitor active lithium loss and phase transitions over the battery's lifecycle.